Multi-Map-Nav-AR100

Introduction:

The Multi-Map-Nav-AR100 package is for navigating the AR100 robot to separate mapped rooms. Each map is mapped in separate sessions, and saved in the maps directory in the Multi_Map_Nav package. Wormhole mechanism allows the robot to switch between maps.

Key Features:

- Switching maps based on Wormhole

- SQLite database to store the map connection coordinates.

- Dynamic Map Switching using map_server

- ROS Action Server for receiving goals

- move_base Integration for local navigation

Scenario Description:

Imagine a robot in am office with multi_rooms. Each room is mapped separately (e.g., reception = map1, office = map2, meeting room = map3).  A user requests the robot to go from reception (map1) to meeting room (map3).

The robot:

- Identifies the wormhole coordinates (map1 → map2 → map3)

- Switch maps as needed

- Continues navigation across maps

System Architecture:

The system consists of three main components that work together to enable seamless navigation across multiple maps:

1. WormholeManager

- Connects to the SQLite database

- Retrieves wormhole coordinates between maps

- Handles queries to find paths between maps

2. MapSwitcher

- Loads map YAML files from the specified directory

- Launches map_server with the appropriate map

- Manages the transition between different environments

3. NavigationServer

- Implements ROS action server for handling navigation goals

- Orchestrates the entire navigation process

- Implements navigation logic for direct and indirect paths

- Utilizes move_base for actual robot movement

How It Works:

A wormhole is a defined position in one map that links to a corresponding position in another map—usually an overlapping area such as a doorway or hallway.

Wormhole Transition Steps:

1. Navigate to the wormhole in the current map.

2. Stop the map_server.

3. Launch map_server with the target map.

4. Teleport the robot to the corresponding wormhole in the new map.

5. Resume navigation toward the final goal.

Navigation Process – Step by Step

1. Goal Receive

The system receives a navigation request specifying:

- Target position (x, y)

- Target map (map_name)

2. Current Map Evaluation

- If the target is within the current map, directly forward the goal to move_base.

- If the target is in another map, initiate multimap path planning.

3. Path Planning Logic

a. Direct Path

Check for a direct wormhole from the current map to the target map. If found,

- Navigate to the wormhole position

- Switch to the destination map

- Continue to the target position

b. Indirect Path (via Central Hub)

If no direct path exists:

- Navigate to a wormhole leading to the central map (map2)

- Switch to map2

- Navigate to a wormhole connecting map2 to the target map

- Switch to the target map

- Navigate to the final target position

4. Move Base Integration

For each segment:

- Send a goal to move_base

- Wait for the result (success/failure)

- If successful, proceed to the next stage

- If failed, abort with an error message

SQL Database Structure:

wormholes.db file creation steps using command line

1. sqlite3

2. sqlite> .open wormholes.db

3. sqlite> CREATE TABLE wormholes_coord (from_map TEXT, to_map TEXT, from_x REAL, from_y REAL);

4. sqlite> INSERT INTO wormholes_coord VALUES ('map1', 'map2', 2.7, 4.9);

5. sqlite> INSERT INTO wormholes_coord VALUES ('map2', 'map1', 2.7, 4.9);

6. sqlite> INSERT INTO wormholes_coord VALUES ('map2', 'map3', 12.3, 6.9);

7. sqlite> INSERT INTO wormholes_coord VALUES ('map3', 'map2', 12.3, 6.9);

8. sqlite> SELECT * FROM wormholes_coord;

The terminal output looks like this
sqlite> SELECT * FROM wormholes_coord;

map1|map2|2.72|4.97

map2|map1|2.72|4.97

map2|map3|12.38|6.9

map3|map2|12.38|6.9

Each row from the database represents a wormhole connection

from_map | to_map | from_x | from_y

Action Definition

NavToGoal.action:

#Request

float64 target_x

float64 target_y

string target_map

---

#Result

bool success

string message

---

#Feedback

string feedback_msg

Folder Structure:

The folder strcture of the Multi_Map_Nav package:

multi_map_nav

.

├── action

│   └── NavToGoal.action

├── database

│   └── wormholes.db

├── include

│   └── multi_map_nav

│       ├── map_switcher.h

│       ├── navigation_server.h

│       └── wormhole_manager.h

├── launch

│   └── multi_map_nav.launch

├── maps

│   ├── map1.pgm

│   ├── map1.yaml

│   ├── map2.pgm

│   ├── map2.yaml

│   ├── map3.pgm

│   └── map3.yaml

├── src

│    ├── map_switcher.cpp

│    ├── navigation_server.cpp

│    └── wormhole_manager.cpp

├── CMakeLists.txt

├── package.xml

└── README.md

Files to launch:

1. Launch the AR100 robot in the simple_world

roslaunch start_anscer start_anscer.launch

2. Launch the localization and navigation server

roslaunch anscer_navigation anscer_navigation.launch

3. Launch the multi_map_nav

roslaunch multi_map_nav multi_map_nav.launch

4. Send a navigation goal:

You can send navigation goals using rostopic:

rostopic pub /navigate_to_goal/goal multi_map_nav/NavToGoalActionGoal "goal:

target_x: 1.5

target_y: 9.0

target_map: 'map1'"

Results:

1. Robot is moving from the Reception(current map) to Office(target map)

2. Robot is moving from the Office(map2) to Meeting room(map3)